Dashboard BOM & Pinout

| Component | Interface | Signal | ESP32 GPIO | Notes |
| --- | --- | --- | --- | --- |
| MAX7219 7-Segment Display | SPI | DIN | GPIO23 | Shared SPI MOSI |
|  | SPI | CLK | GPIO18 | Shared SPI SCK |
|  | SPI | CS | GPIO5 | Dedicated Chip Select |
|  | Power | VCC | 5V |  |
|  | Power | GND | GND |  |
| nRF24L01+ | SPI | MOSI | GPIO23 | Shared SPI |
|  | SPI | MISO | GPIO19 | Shared SPI |
|  | SPI | SCK | GPIO18 | Shared SPI |
|  | GPIO | CE | GPIO4 |  |
|  | GPIO | CSN | GPIO15 | Dedicated Chip Select |
|  | Power | VCC | 3.3V ONLY | Never connect to 5V |
|  | Power | GND | GND |  |
|  | Optional | IRQ | NC | Not connected |

Controller BOM & Pinout

| Component | Interface | Signal | ESP32 GPIO | Notes |
| --- | --- | --- | --- | --- |
| HX711 | GPIO | DT (DOUT) | GPIO34 | Input only |
|  | GPIO | SCK | GPIO25 |  |
|  | Power | VCC | 5V / 3.3V* | Module dependent |
|  | Power | GND | GND |  |
| SH1106 OLED | I²C | SDA | GPIO21 | Address 0x3C |
|  | I²C | SCL | GPIO22 |  |
|  | Power | VCC | 3.3V / 5V | Depends on module |
|  | Power | GND | GND |  |
| 4×4 Keypad | Matrix | Row0 | GPIO32 |  |
|  |  | Row1 | GPIO33 |  |
|  |  | Row2 | GPIO26 |  |
|  |  | Row3 | GPIO27 |  |
|  |  | Col0 | GPIO12 | Strapping pin |
|  |  | Col1 | GPIO13 |  |
|  |  | Col2 | GPIO14 |  |
|  |  | Col3 | GPIO15 | Strapping pin |
| nRF24L01+ | SPI | MOSI | GPIO23 | Shared SPI |
|  | SPI | MISO | GPIO19 |  |
|  | SPI | SCK | GPIO18 |  |
|  | GPIO | CE | GPIO16 |  |
|  | GPIO | CSN | GPIO17 |  |
|  | Power | VCC | 3.3V ONLY |  |
|  | Power | GND | GND |  |

Options

1. ADS1232 (Alternative ADC)

Instead of HX711

| Signal | GPIO |
| --- | --- |
| DOUT | GPIO34 |
| SCLK | GPIO25 |
| PDWN | GPIO26 |
| SPEED | GPIO27 |
| A0 | GPIO14 |

Can’t be used with 4x4 keypad without pc8574

2. MAX7219 7-Segment Display (Optional Display)

Instead of the OLED.

| Signal | GPIO |
| --- | --- |
| DIN | GPIO23 |
| CLK | GPIO18 |
| CS | GPIO5 |
| VCC | 5V |
| GND | GND |

Shares the SPI bus with the nRF24.

3. Multiple Load Cells

The firmware supports:

- 1 Load Cell (current)

- 2 Load Cells

- 4 Load Cells

For multiple load cells, each requires its own dedicated ADC channel/pin set, and the arrays in pins.h must be expanded accordingly.